BILCO Bil-Guard 2.0 fixed hatch railing system provides a permanent means of fall protection for roof hatch openings. System meets and exceeds OSHA fall protection regulations (29 CFR 1910.23). Bil-Guard® models are available for most roof hatch and automatic fire vent sizes. For more information, please visit our website www.bilco.com or call 1-800-366-6530.

SECTION 077233

ROOF HATCH rail system

(BILCO TYPE BIL-GUARD 2.0)

- GENERAL

      - SUMMARY

        - Work Included:  Provide factory-fabricated fixed hatch railing system.

      - SUBMITTALS

        - Product Data:  Submit manufacturer’s product data.

        - Shop Drawings: Submit shop drawings including profiles, accessories, location, adjacent construction interface, and dimensions.

        - Warranty:  Submit executed copy of manufacturer’s standard warranty.

      - QUALITY ASSURANCE

        - Manufacturer:  A minimum of 5 years experience manufacturing similar products.

        - Installer:  A minimum of 2 years experience installing similar products.

        - Manufacturer’s Quality System: Registered to ISO 9001:2008 Quality Standards including in-house engineering for product design activities.

      - DELIVERY, STORAGE AND HANDLING

        - Deliver products in manufacturer’s original packaging.  Store materials in a dry, protected, well-vented area. Inspect product upon receipt and report damaged material immediately to delivering carrier and note such damage on the carrier’s freight bill of lading.

      - WARRANTY

        - Manufacturer’s Warranty:  Provide manufacturer’s standard warranty. Materials shall be free of defects in material and workmanship for a period of five years from the date of purchase. Should a part fail to function in normal use within this period, manufacturer shall furnish a new part at no charge.

- PRODUCTS

      - MANUFACTURER

        - Basis-of-Design Manufacturer:  Type Bil-Guard® 2.0 Roof Hatch Railing System by The BILCO Company, P.O. Box 1203, New Haven, CT 06505, 800-366-6530, Fax: 203-933-8478, Web: www.bilco.com.

      - HATCH RAIL SYSTEM

        - Furnish and install where indicated on plans hatch rail system Model [insert RL2-S; RL2-NB; RL2-L; RL2-E; RL2-F, RL2-SS, RL2-D].  The hatch rail system shall be field assembled and installed (by others) per the manufacturer’s instructions.

        - Performance characteristics:

          - High visibility safety yellow powder coat paint finish (other colors available as a special order).

          - Hatch rail system shall attach to the capflashing of the roof hatch and shall not penetrate any roofing material.

          - Hatch rail system shall satisfy the requirements of OSHA 29 CFR 1910.23 and shall meet OSHA strength requirements with a factor of safety of two.

          - Corrosion resistant construction with a five-year warranty.

          - Hinged gate shall ensure continuous barrier around the roof hatch.

          - Self-closing gate hinge and positive latching system provided with hatch rail system.

        - Posts and Rails:  1-1/4” (32mm) 6061 T6 schedule 40 aluminum pipe

        - Hardware:  Mounting brackets shall be 3/8” (9mm) thick extruded aluminum.  Pivoting post guides with compression fittings and latching mechanism shall be cast aluminum. Self-closing hinges and all fasteners shall be type 316 stainless steel.

- EXECUTION

      - EXAMINATION

        - Examine substrates and openings for compliance with requirements for installation tolerances and other conditions affecting performance.  Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION

        - Install products in strict accordance with manufacturer’s instructions and approved submittals.  Locate units level, plumb, and in proper alignment with adjacent work.

          - Test units for proper function and adjust until proper operation is achieved.

          - Repair finishes damaged during installation.

          - Restore finishes so no evidence remains of corrective work.

      - ADJUSTING AND CLEANING

        - Clean exposed surfaces using methods acceptable to the manufacturer which will not damage finish.

END OF SECTION