Bilco Type KD double leaf floor access doors are generally for interior use or exterior applications where water or liquid infiltration into the access opening is not a concern. Type KD floor access doors feature aluminum construction, 150 PSF reinforcing for pedestrian applications, and engineered lift assistance for easy one-hand operation. Available in a number of standard sizes and special sizes can be specified to meet unique access requirements. For more information, please visit our website www.bilco.com or call 1-800-366-6530.

SECTION 083100

FLOOR ACCESS DOORS

(BILCO TYPE KD)

- GENERAL

      - SUMMARY

        - Work Included:  Provide factory-fabricated floor access doors.

      - SUBMITTALS

        - Product Data:  Submit manufacturer’s product data.

        - Shop Drawings:  Submit shop drawings including profiles, accessories, location, adjacent construction interface, and dimensions.

        - Warranty:  Submit executed copy of manufacturer’s standard warranty.

      - QUALITY ASSURANCE

        - Manufacturer:  A minimum of 5 years experience manufacturing similar products.

        - Installer:  A minimum of 2 years experience installing similar products.

        - Manufacturer’s Quality System: Registered to ISO 9001:2008 Quality Standards including in-house engineering for product design activities.

      - DELIVERY, STORAGE AND HANDLING

        - Deliver products in manufacturer’s original packaging. Store materials in a dry, protected, well-vented area. Inspect product upon receipt and report damaged material immediately to delivering carrier and note such damage on the carrier’s freight bill of lading.

      - WARRANTY

        - Manufacturer’s Warranty:  Provide manufacturer’s standard warranty.  Materials shall be free of defects in material and workmanship for a period of five years from the date of purchase. Should a part fail to function in normal use within this period, manufacturer shall furnish a new part at no charge.

- PRODUCTS

      - manufacturer

        - Basis-of-Design Manufacturer:  Type KD Floor Access Door by The Bilco Company, P.O. Box 1203, New Haven, CT 06505, 1-800-366-6530, Fax: 1-203-933-8478, Web: www.bilco.com.

      - ACCESS DOOR

        - Furnish and install where indicated on plans vault access door Type KD, size [insert: width (_____) x length (_____)]. Length denotes hinge side. The floor access door shall be double leaf and pre-assembled from the manufacturer.

        - Performance characteristics:

          - Covers: Shall be reinforced to support a minimum live load of 150 psf (732 kg/m2) with a maximum deflection of 1/150th of the span.

          - Operation of the covers shall be smooth and easy with controlled operation throughout the entire arc of opening and closing.

          - Operation of the covers shall not be affected by temperature.

        - Covers: Shall be 1/4” (6mm) aluminum diamond pattern plate.

        - Frame: Shall be extruded aluminum with strap anchors bolted to the exterior.

        - Hinges: Shall be specifically designed for horizontal installation and shall be bolted to the underside of cover.

        - Lifting mechanisms: Cam-action hinges shall pivot on torsion bars to provide, smooth, easy, and controlled cover operation throughout the entire arc of opening and to act as a check in retarding downward motion of the cover when closing.

        - A removable exterior turn/lift handle with a spring loaded ball detent shall be provided to open the cover.

        - Hardware:

          - Hinges: Cast steel cam-action hinges which pivot on torsion bars shall be provided.

          - Covers shall be equipped with steel hold open arms that automatically lock each cover in the open position.

          - Covers shall be fitted with the required number and size of torsion bars.

          - A Type 316 stainless steel snap lock with fixed handle shall be mounted on the underside of the covers.

          - Hardware: Shall be zinc plated and chromate sealed. Type 316 stainless steel hardware is available for installation in corrosive environments.

        - Finishes: Factory finish shall be mill finish aluminum with bituminous coating applied to the exterior of the frame.

- EXECUTION

      - EXAMINATION

        - Examine substrates and openings for compliance with requirements for installation tolerances and other conditions affecting performance.  Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION

        - Install products in strict accordance with manufacturer’s instructions and approved submittals.  Locate units level, plumb, and in proper alignment with adjacent work.

          - Test units for proper function and adjust until proper operation is achieved.

          - Repair finishes damaged during installation.

          - Restore finishes so no evidence remains of corrective work.

      - ADJUSTING AND CLEANING

        - Clean exposed surfaces using methods acceptable to the manufacturer which will not damage finish.

END OF SECTION