Bilco Type TER floor access doors allow design professionals to provide concealed access in decorative flooring materials. The pan-type cover will accept a wide variety of architectural materials, allowing the doors to be used in virtually any application. Whether it’s ceramic tile, hardwood, or even granite, each TER door is precisely engineered with lift assistance to afford the user with easy, one-hand operation. Type TER floor access doors feature aluminum construction and are available in a number of standard sizes. Special sizes can also be specified for unique access requirements. For more information, please visit our website www.bilco.com or call 1-800-366-6530.

SECTION 083100

FLOOR ACCESS DOORS

(BILCO TYPE TER)

- GENERAL

      - SUMMARY

        - Work Included:  Provide factory-fabricated floor access doors.

      - SUBMITTALS

        - Product Data:  Submit manufacturer’s product data.

        - Shop Drawings: Submit shop drawings including profiles, accessories, location, adjacent construction interface, and dimensions.

        - Warranty:  Submit executed copy of manufacturer’s standard warranty.

      - QUALITY ASSURANCE

        - Manufacturer:  A minimum of 5 years experience manufacturing similar products.

        - Installer:  A minimum of 2 years experience installing similar products.

        - Manufacturer’s Quality System: Registered to ISO 9001:2008 Quality Standards including in-house engineering for product design activities.

      - DELIVERY, STORAGE AND HANDLING

        - Deliver products in manufacturer’s original packaging. Store materials in a dry, protected, well-vented area. Inspect product upon receipt and report damaged material immediately to delivering carrier and note such damage on the carrier’s freight bill of lading.

      - WARRANTY

        - Manufacturer’s Warranty: Provide manufacturer’s standard warranty. Materials shall be free of defects in material and workmanship for a period of five years from the date of purchase. Should a part fail to function in normal use within this period, manufacturer shall furnish a new part at no charge.

- PRODUCTS

      - MANUFACTURER

        - Basis-of-Design Manufacturer: Type TER Floor Access Door by The Bilco Company, P.O. Box 1203, New Haven, CT 06505, 1-203-934-6363, Fax: 1-203-933-8478, Web: www.bilco.com.

      - ACCESS DOOR

        - Furnish and install where indicated on plans vault access door TER, size [insert: width (_____) x length (_____)]. Length denotes hinge side. The floor access door shall be single leaf and pre-assembled from the manufacturer.

        - Performance characteristics:

          - Cover: Shall be reinforced to support a minimum live load of 150 psf (732kg/m2) with a maximum deflection of 1/150th of the span.

          - Operation of the cover shall be smooth and easy with controlled operation throughout the entire arc of opening and closing.

          - Operation of the cover shall not be affected by temperature.

        - Cover shall have a 1” (25mm) fillable pan to receive concrete or a combination of concrete and [insert specified flooring material, including type, thickness, and weight per ft2 (m2) _______]. All fill material to be furnished and installed by others in the field.

        - Frame: Shall be extruded aluminum with full anchor flange around the perimeter.

        - Lifting mechanisms: Manufacturer shall provide the required number and size of compression spring operators enclosed in telescopic tubes to provide, smooth, easy, and controlled cover operation throughout the entire arc of opening and to act as a check in retarding downward motion of the cover when closing. The upper tube shall be the outer tube to prevent accumulation of moisture, grit, and debris inside the lower tube assembly. The lower tube shall interlock with a flanged support shoe fastened to a formed 1/4” (6mm) gusset support plate.

        - A removable exterior turn/lift handle with a spring loaded ball detent shall be provided to open the and the latch release shall be protected by a flush, gasketed, removable screw plug.

        - Hardware:

          - Hinges: Shall be a continuous heavy duty Type 316 stainless steel hinge that is accessible only when the cover is in the open position.

          - Cover shall be equipped with an aluminum hold open arm that automatically locks the cover in the open position.

          - Cover shall be fitted with the required number and size of compression spring operators.

          - A Type 316 stainless steel snap lock with fixed handle shall be mounted on the underside of the cover.

          - Hardware: Compression spring tubes shall be an anti-corrosive composite, all fasteners shall be Type 316 stainless steel material, and all other hardware shall be zinc plated and chromate sealed. [For installation in highly corrosive environments or when prolonged exposure to hot water or steam is anticipated, specify Type 316 stainless steel hardware].

        - Finishes: Factory finish shall be mill finish aluminum with bituminous coating applied to the exterior of the frame.

- EXECUTION

      - EXAMINATION

        - Examine substrates and openings for compliance with requirements for installation tolerances and other conditions affecting performance.  Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION

        - Install products in strict accordance with manufacturer’s instructions and approved submittals.  Locate units level, plumb, and in proper alignment with adjacent work.

          - Test units for proper function and adjust until proper operation is achieved.

          - Repair finishes damaged during installation.

          - Restore finishes so no evidence remains of corrective work.

      - ADJUSTING AND CLEANING

        - Clean exposed surfaces using methods acceptable to the manufacturer which will not damage finish.

END OF SECTION